SHAWLOWS RULER

Read handout "Measuring the Wavelength of Light with a Ruler" by A. L. Schawlow.  Other handouts and discussion may be posted.

Tasks.

Use the HeNe laser (Wavelength = 6328.1646 Å in air  ) and large steel rule as described in the handout to measure interference fringe positions.  The reflecting surface (a long and very rigid ruler) should be placed reasonably far from a wall and made level.  You will tape together a strip of paper on the wall (far away) to record data. Make sure you have made all measurements before removing (distance from spot to wall and both straight through spot and reflected zero order).  You will record fringe positions on the paper.  You will need to carefully hold the laser and reflecting steel rule in place to make measurements.  You will need to know which set of rulings you are reflecting light from and how far the center of the beam on the ruler –from the wall.  You will also need to identify the zero order (direct) reflection as well as a straight through shot of the laser hitting the wall.  Give careful thought to the geometry on this setup..

Notes.  The reflection interference pattern may have visible secondary maxima as well as principal maxima.  With the laser fixed, you should be able to use the translator on the laser to determine a “straight through” position.  You should also be able to reflect the laser off a smooth portion of the steel to determine the “center” fringe position.  The positions you mark should be the center of the primary portion of the bright fringe spots (not secondary interference fringes).  Measure positions of the center of the principal maxima from n=0 to as high an order as you can.  It can be useful to ensure that you have as much data as you MIGHT need, even if you decide not to use all of that data).  Take care to insure that the initial angle of the zero order fringe is relatively small.  You need to be able to measure the distance from the point where reflection occurs to the wall.

Tasks:

1.	Use at least two sets of rulings so that you have two data sets of fringe position vs. 	number.  If you slide the ruler in and out the beam will hit different rulings, or 	you may use those strange numbers on the OTHER SIDE.

2.	You should determine how to fit a curve to your data to determine the 	wavelength. (there are several choices to consider for doing this).

3.	In your reports have both plots with fit information.

4.  	Discuss uncertainty and compare to known wavelength.

5.	Consider and discuss approximations used and validity.

6.	Make sure you carefully describe the experimental setup and label what 	parameters are being measured.