The Michelson Interferometer

Note several adjustments that you can make with this instrument as demonstrated.  There is a height adjustment knob.  Note the mirror translation scheme has two knobs, a fine tuning knob and gross adjustment.  The fine tuning knob may be used when the gear is in contact with the gear for the gross adjustment knob.  Turn the gross adjustment knob only when the fine tuning knob is disengaged.  Lift up on the fine adjust knob arm to disengage this arm.  The (an) instrument documentation manual (long) is posted for your use.  You may also want to browse either the internet or any standard optics text for discussion of the Michelson interferometer.  A Michelson interferometer is the heart of many high precision instruments such as Fourier Transform Interferometers.  Processes that follow from division of amplitude (beam splitting) interference allow for high precision measurements of lengths, wavelength, and development of high quality optical cavities.

The translating mirror has mm markings along the translation arm of the interferometer.  The gross adjust knob has smallest markings of one hundredth of a millimeter.  I expect readings to one tenth of the smallest division.  Note that this smallest division marking is still about twenty times the wavelength of typical visible light.  Nonetheless the fine adjustment knob allows for easy accurate control at the level of single fringes (which indicates control at the half wavelength of light level)!

I have placed a lens in the HeNe laser path to the interferometer to expand the beam and give a more extended source.  You should use this setup to become familiar with the instrument.  You may adjust the position of the free standing lens, the position of the interferometer and any knobs on the interferometer.  You are responsible for being able to perform a quick and accurate setup.

DO NOT TOUCH OR ATTEMPT TO CLEAN THE MIRRORS.

TASKS

A.)	Set up the Michelson Interferometer so that the bright HeNe laser passes through the free standing lens and into the interferometer.  After careful alignment you should see a "bullseye" interference pattern on the wall.  Each of you should be able to set up the instrument in a few minutes. You will be required to show me this!  Be very careful using the lenses and other optics with this laser.  Remove jewelry and make sure you know where any stray rays are directed.  With the interference pattern set up count fringes vs. mirror displacement to determine the wavelength of the HeNe laser.  Make sure you count at least a few hundred fringes (think about precision). You may want to repeat this measurement a few times for a good average. In your report make sure you discuss any relevant uncertainties in your readings and results.  Note that the mirror movement has "play" or "slop" in the gear mechanisms, so you should always come from low to high when taking measurements. You should note the significance of being near or far from the very special “equal path length condition”.

In the HeNe Laser path I use an external lens next to the laser to make the beam diverge.  This helps to enlarge the interference pattern.  The lens on the interferometer may or may not be used in addition.

B.)	Use the sodium lamp to view a pattern directly (you must look at viewing into the interferometer into the light (sodium light---not laser).  I have had success shining the yellow light on paper and then into the interferometer (carefully directed). The pattern is viewed while your eye is focused at infinity. You may move the interferometer and you do not need the free standing lens—place the sodium lamp in front of the interferometer. Measure the distance that the travelling arm must travel between each of several consecutive wash-out positions (lowest contrast pattern positions).  These are the positions where the interference pattern is least distinct.  You need several such readings and you need to make sure that you do not confuse a washout position with the very special “equal path conditions” (for that condition the fringe is very broad, covers a large region and looks like washout….but it is not).  You viewed that equal path length position in part A—so you know about it.   Don’t get confused on counting these nearly equally spaced washout conditions.

These washouts occur due to two different overlapping interference bullseye patters.   The patterns come from two different closely spaced resonant emission lines in sodium.  When the bright rings from one pattern lie halfway between the bright rings of the other pattern, the entire pattern becomes indistinct (washouts).  You will measure how far to move the adjustable mirror between washouts (plot mirror position vs. washout number).

This information can be used to determine the wavelength spacing between the two sodium lines    (588.9950 nm and 589.5924nm in air).  The average of the two wavelengths  is 589.1941nm.  In order to determine the wavelength spacing consider the following.  At the first washout position (call this washout A), the interference condition for the center of the pattern for the longer wavelength is roughly      2dA=m1A1 .  Now for the other wavelength (say the shorter) this same spacing corresponds to some different whole number of wavelengths plus a half wavelength for the shorter wavelength.  So the distance 2dA =(m2A+1/2 )2 . The two different values for m's are both integers so the two differ from each other by some integer value n.  That is since both are integers then m1A+n=m2A for some integer n. The path length for a particular washout position is then 2dA=m1A1=(m1A+n+1/2 )2  .   The "A" subscript refers to this particular washout at this path difference.

Now if d is increased to the next washout condition then the fringe number for the shorter wavelength has increased by one more fringe than for the longer wavelength.  (As the path difference is changed then more of the shorter wave fringes pass by than for the longer, it is one more by the next washout position).

So the new condition is 2dB= m1B1=(m1B+n+3/2)2.  You can use these conditions to obtain an approximate expression for   in terms of measurements.  Note 12(ave)2 since the two wavelengths are close together.  Compare your measured separation of the two sodium lines to the known.  Discuss uncertainties in your measurements.

C.) 	Demonstrate to me that you can find white-light fringes.  You will measure where that condition is located.  You should find white light fringes by any means possible and demonstrate these to me (to verify the effect).  Then expect me to mis-align the interferometer, and you should find the white light fringes within one hour.  This is your hands on portion of the lab.

D) 	There are many other things that can be done with this type of setup.  Different laser (see me), index of air, other…..