Scan Sheet

| Line | Wavelength Range | PMT | Lockin Sens | Scan Rate |
| --- | --- | --- | --- | --- |
| Zero Sodium | -0.5 to 0.5nm | 0.200kV | 500nA | 1nm/min |
| Sodium-both | 587-591nm | 0300kV | 1.00µA | 1nm/min |
| Zero Hydrogen | -0.5 to 0.5nm | 0.400kV | 200nA | 1nm/min |
| Hydrogen~434 | 433-435nm | 1.000kV | 1.000µA | 1nm/min |
| Hydrogen~486 | 484-490nm | 0.700kV | 500nA | 2nm/min |
| Hydrogen~656 | 654-658nm | 0.700kV | 500nA | 2nm/min |

Other settings to note.

Lens=25cm fl.

Monochromator is at distance  50cm from lens, light source is at 50 cm from lens.

Chopper frequency 270 to 280 Hz

Monochromator slits 50 µm –entry and exit

Lockin Amplifier

I x 106 ---front panel setting

DC, Float, High Res, Both filters in, “Display”, Sin, Phase.

With signal (set monochrmator positions to    -0.100nm)—you must push “auto phase” button

Room lights---low light livel.

The settings in the table for PMT, Lockin Sensitivity scale, and scan rate are things you may adjust.  If you observe a “flat top peak” you should lower the PMT voltage.  If you observe pixilation (digital bit level) on data acquisition throughout the entire scan—you should zoom in to a lower sensitivity scale on the lockin.  The values in the table are empirical and worked—however, any motion of light source, lens, monochromator (I mean by small amounts) can change those settings.  It is OK to correct settings and scan again.