Scan---Determining “zero” offset of monochromator---USE Sodium Lamp

Monochromator and PMT settings

- PMT Power Supply Voltage=____________kV

- Entry and Exit Slit size set=_______________m

- Lens Distance

- Light Source Distance—approx

- Image centered on slits—check.

- Lamp warmed up

Computer Scan Settings

Computer login and password and program

- Scan Sample rate= 10 samples/second

- Scan from = -1nm

- Scan to= 1nm

- Scan Rate=1nm/minute

- Check display on monochromator controller to ensure set to start and scanning
(if needed reboot monochromator and the software on computer)

Lockin Amplifier Settings

- Chopper at ~270 to 280 Hz

- Sensitivity

- Time Constant

- X or R

- Other settings

After saving your scan you should open origin and make a quick plot to view----This ensures your scan is correct and has a well defined peak for your settings.